1. LED ON Using Arduino UNO in Proteus 8

void setup()

{

pinMode(13, OUTPUT);   // Set digital pin 13 as output

}

void loop()

{

digitalWrite(13, HIGH); // Turn ON LED

}

2. Blinking both internal and external Light Emitting Diode

(LED) using Arduino UNO and Interfacing switches with Arduino

UNO

| Component Name (Proteus) | Quantity |
| --- | --- |
| Arduino UNO R3 | 1 |
| LED-RED | 1 |
| Resistor 220Ω | 1 |
| Push Button | 1 |
| Resistor 10kΩ | 1 |
| Ground (GND) | 1 |

int extLED = 12;      // External LED pin

int button = 2;      // Switch pin

void setup()

{

pinMode(13, OUTPUT);      // Internal LED

pinMode(extLED, OUTPUT); // External LED

pinMode(button, INPUT);  // Switch input

}

void loop()

{

if (digitalRead(button) == HIGH)

{

digitalWrite(13, HIGH);

digitalWrite(extLED, HIGH);

delay(500);

digitalWrite(13, LOW);

digitalWrite(extLED, LOW);

delay(500);

}

else

{

digitalWrite(13, LOW);

digitalWrite(extLED, LOW);

}

}

3. Design and implement Interfacing switches with Arduino

UNO

| Component Name | Quantity |
| --- | --- |
| Arduino UNO R3 | 1 |
| Push Button Switch | 1 |
| LED-RED | 1 |
| Resistor 220Ω | 1 |
| Resistor 10kΩ | 1 |
| Ground (GND) | 1 |

int led = 13;      // Internal LED

int button = 2;   // Switch pin

void setup()

{

pinMode(led, OUTPUT);

pinMode(button, INPUT);

}

void loop()

{

if (digitalRead(button) == HIGH)

{

digitalWrite(led, HIGH);   // LED ON

}

else

{

digitalWrite(led, LOW);    // LED OFF

}

}

4. Design and implement interfacing a 7-segment display with Arduino UNO

| Component Name (Proteus) | Quantity |
| --- | --- |
| Arduino UNO R3 | 1 |
| 7-Segment Display (Common Cathode) | 1 |
| Resistor 220Ω | 7 |
| Ground (GND) | 1 |

// Common Cathode 7-Segment Display

int segPins[7] = {2, 3, 4, 5, 6, 7, 8};

// Segment patterns for 0–9 (a b c d e f g)

int numbers[10][7] = {

{1,1,1,1,1,1,0}, // 0

{0,1,1,0,0,0,0}, // 1

{1,1,0,1,1,0,1}, // 2

{1,1,1,1,0,0,1}, // 3

{0,1,1,0,0,1,1}, // 4

{1,0,1,1,0,1,1}, // 5

{1,0,1,1,1,1,1}, // 6

{1,1,1,0,0,0,0}, // 7

{1,1,1,1,1,1,1}, // 8

{1,1,1,1,0,1,1}  // 9

};

void setup()

{

for (int i = 0; i < 7; i++)

{

pinMode(segPins[i], OUTPUT);

}

}

void loop()

{

for (int num = 0; num <= 9; num++)

{

for (int seg = 0; seg < 7; seg++)

{

digitalWrite(segPins[seg], numbers[num][seg]);

}

delay(1000);  // 1 second delay

}

}

5. Design and Implementation of Interfacing Multiple 7-Segment Displays with Arduino UNO (Proteus 8)

| Component | Quantity |
| --- | --- |
| Arduino UNO R3 | 1 |
| 7-Segment Display (Common Cathode) | 2 |
| Resistor 220Ω | 7 |
| Resistor 1kΩ | 2 |
| Ground (GND) | 1 |

// 1st Display (Pins 13 to 7)

int disp1[7] = {13,12,11,10,9,8,7};

// 2nd Display (Pins 6 to 0)

int disp2[7] = {6,5,4,3,2,1,0};

// Segment pattern for Common Cathode (a b c d e f g)

int numbers[10][7] = {

{1,1,1,1,1,1,0}, // 0

{0,1,1,0,0,0,0}, // 1

{1,1,0,1,1,0,1}, // 2

{1,1,1,1,0,0,1}, // 3

{0,1,1,0,0,1,1}, // 4

{1,0,1,1,0,1,1}, // 5

{1,0,1,1,1,1,1}, // 6

{1,1,1,0,0,0,0}, // 7

{1,1,1,1,1,1,1}, // 8

{1,1,1,1,0,1,1}  // 9

};

void setup()

{

for(int i=0;i<7;i++)

{

pinMode(disp1[i], OUTPUT);

pinMode(disp2[i], OUTPUT);

}

}

void loop()

{

for(int num = 0; num < 100; num++)

{

int tens = num / 10;

int ones = num % 10;

for(int i=0;i<7;i++)

{

digitalWrite(disp1[i], numbers[tens][i]);

digitalWrite(disp2[i], numbers[ones][i]);

}

delay(1000); // 1 second delay

}

}

6.Implementation and Design of ADC and Analog Temperature Sensor

Using Arduino.

#include<LiquidCrystal.h>

LiquidCrystal lcd(12,11,5,4,3,2);

void setup() {

// put your setup code here, to run once:

lcd.begin(16,2);

}

void loop() {

// put your main code here, to run repeatedly:

int reading = analogRead(A0);

float voltage = reading*(5.0/1023.0);

float tempC = voltage * 100;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("TEMP: ");

lcd.setCursor(0, 1);

lcd.print(String(tempC));

delay(1000);

}

7. Design and implement a system to detect darkness/light using LDR using Arduino

#include<LiquidCrystal.h>

LiquidCrystal lcd(12,11,5,4,3,2);

int ldr_port = A0;

void setup() {

// put your setup code here, to run once:

lcd.begin(16,2);

pinMode(7, OUTPUT);

}

void loop() {

// put your main code here, to run repeatedly:

int reading = analogRead(ldr_port);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("LDR: ");

lcd.setCursor(0, 1);

lcd.print(String(reading));

delay(1000);

}

8. Design and Implement 16*2 LCD Display to print “Welcome To CSE, FEC” using Arduino give me step by step procedure for lab using proteus 8

#include<LiquidCrystal.h>

LiquidCrystal lcd(12,11,5,4,3,2);

void setup(){

lcd.begin(16,2);

}

void loop(){

lcd.clear();

lcd.setCursor(0,0);

lcd.print("Welcome to CSE");

lcd.setCursor(6, 1);

lcd.print("FEC");

delay(1000);

}